# Exercise 2

# MES Scheduling for DIBRIS-BIKE

## 1. Introduction

In this project, we implemented a small part of the MES (Manufacturing Execution System) for the company DIBRIS-BIKE.

In the considered part of the plant, the cut tubes arrive in an accumulation area, and then they must be processed by two machines:

- Welding machine

- Oven

The goal of the exercise is to determine the best order of processing the tubes in order to minimize the total production time (makespan).

To solve this problem, we used the Johnson scheduling algorithm, which is suitable for systems with two machines.

The tools used in this project are:

- SQL Server for storing the data

- MATLAB for running the scheduling algorithm

- Docker / Azure SQL to run SQL Server on a Mac system

# 2. Database Design

First, a database called:

DIBRIS_BIKE

was created in SQL Server.

Two main tables were designed.

## 2.1 Table: cutted_tubes

This table stores the information about the tubes located in the accumulation area.

The table contains the following attributes:

- id → unique identifier of the tube

- batch_id → batch identifier

- processing_time_on_welding → processing time on the welding machine

- processing_time_on_oven → processing time on the oven

This table represents the input data for the scheduling problem.

## 2.2 Table: johnson_schedule

This table stores the results produced by the Johnson algorithm.

The stored information includes:

- tube_id → identifier of the tube

- sequence_pos → position of the tube in the final sequence

- machine1_start / machine1_end → start and end time on the welding machine

- machine2_start / machine2_end → start and end time on the oven

- makespan → total production time

This table represents the final production schedule.

# 3. Reading Data from SQL

In MATLAB, the program first connects to the SQL Server database.

Then the tubes belonging to a specific batch are retrieved.

Example SQL query:

SELECT id, processing_time_on_welding, processing_time_on_oven

FROM dbo.cutted_tubes

WHERE batch_id = 1

The retrieved data are stored in two vectors:

- A → processing time on the welding machine

- B → processing time in the oven

These vectors are used as input for the scheduling algorithm.

# 4. Johnson Scheduling Algorithm

To solve the scheduling problem, we implemented Johnson’s rule.

This algorithm is designed for two-machine flow shop problems.

The algorithm works as follows:

- Find the smallest processing time among all jobs.

- If the shortest time belongs to machine 1, place that job at the beginning of the sequence.

- If the shortest time belongs to machine 2, place that job at the end of the sequence.

- Repeat the process until all jobs are scheduled.

The result is an optimal processing sequence.

# 5. Timing Calculation

After the Johnson sequence is determined, the start and end times on both machines are calculated.

For machine 1:

start(i) = end(i-1)

end(i) = start(i) + processing_time

For machine 2:

start2(i) = max(end1(i), end2(i-1))

end2(i) = start2(i) + processing_time

The makespan is the finishing time of the last job on the second machine.

# 6. Storing Results in SQL

After calculating the schedule, the results are stored in the table:

johnson_schedule

For each tube, the following information is saved:

- sequence position

- start and end times on each machine

- overall makespan

This allows the MES system to store and monitor the production schedule.

# 7. Verification of the Algorithm

To verify the correctness of the Johnson algorithm, the result was compared with the exact optimal solution.

The optimal solution was obtained using a brute-force method:

- All possible job sequences were generated.

- The makespan of each sequence was calculated.

- The best sequence was selected.

Then the Johnson result was compared with the optimal result.

The experiments showed that:

Johnson makespan = Optimal makespan

This confirms that the implementation works correctly.

# 8. Testing with Different Instances

Finally, the algorithm was tested using different batches of tubes.

For each batch:

- Data were read from SQL Server

- Johnson’s scheduling was executed

- The results were stored in the database

- The makespan was analyzed

These tests confirmed that the system works correctly for different problem instances.

# 9. Conclusion

In this project, a small MES component for the DIBRIS-BIKE production system was implemented.

The main steps of the project were:

- Designing the database in SQL Server

- Reading production data in MATLAB

- Implementing the Johnson scheduling algorithm

- Computing the production timing

- Storing the schedule in the database

- Verifying the results using an exact solution

The results show that the Johnson algorithm successfully finds the optimal sequence for the two-machine scheduling problem and minimizes the production makespan.